ASSIGNMENT-5

- Software user input to hardware action

Code:

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

HAL_UART_Receive(&huart1, UART1_rxBuffer,12,10000);

HAL_UART_Transmit(&huart1,UART1_rxBuffer,sizeof(UART1_rxBuffer),10000);

if(*UART1_rxBuffer=='1'){

HAL_GPIO_WritePin(led_GPIO_Port, led_Pin, 1);

HAL_GPIO_WritePin(led2_GPIO_Port, led2_Pin, 0);

HAL_UART_Transmit(&huart1, (uint8_t *)"led1 on!", 10,10000);

}

if(*UART1_rxBuffer=='2') {

HAL_GPIO_WritePin(led_GPIO_Port, led_Pin, 1);

HAL_GPIO_WritePin(led2_GPIO_Port, led2_Pin, 1);

HAL_UART_Transmit(&huart1, (uint8_t *)"led1 and led2  on!", 30,10000);

}

if(*UART1_rxBuffer=='3'){

HAL_GPIO_WritePin(led_GPIO_Port, led_Pin, 0);

HAL_GPIO_WritePin(led2_GPIO_Port, led2_Pin, 0);

HAL_UART_Transmit(&huart1, (uint8_t *)"led1 and led2  off!", 20,10000);

}

if(*UART1_rxBuffer=='4'){

while(1){

HAL_GPIO_WritePin(led_GPIO_Port, led_Pin, 1);

HAL_Delay(1000);

HAL_GPIO_WritePin(led_GPIO_Port, led_Pin, 0);

HAL_GPIO_WritePin(led2_GPIO_Port, led2_Pin, 1);

HAL_Delay(1000);

HAL_GPIO_WritePin(led2_GPIO_Port, led2_Pin, 0);

HAL_UART_Transmit(&huart1, (uint8_t *)"Invalid Input!", 20,10000);

}

}

HAL_UART_Transmit(&huart1, UART1_endBuffer, sizeof(UART1_endBuffer),10000);

}

Configuration:

Ouput:

From terra terminal

Output on MCU :

- Morse code:

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

//HAL_UART_Transmit(&huart1,input , sizeof(input), 10000);

//HAL_UART_Transmit(&huart1,"enter name in upper case ", 50, 10000);

//HAL_Delay(6000);

HAL_UART_Receive_DMA(&huart1, input, 5000);

//HAL_UART_Transmit(&huart1,input , sizeof(input), 10000);

//HAL_UART_Transmit(&huart1,"Entercharacter" , 40, 10000);

HAL_Delay(5000);

ledMorseAlphabet(input[i]);

/*space(); // 3 morse unit time space between 2 initials

space();

space();*/

i=i+1;

}

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart) {

HAL_UART_Receive_DMA(&huart1, input, 10000);

}

void dot() {

HAL_UART_Transmit(&huart1, " . ", 3, 1000);//Serial output to show user, current state

HAL_GPIO_WritePin(led_morse_GPIO_Port, led_morse_Pin, GPIO_PIN_SET);

HAL_Delay(1000);

HAL_GPIO_WritePin(led_morse_GPIO_Port, led_morse_Pin, GPIO_PIN_RESET);

HAL_Delay(1000);

return;

}

void dash() {

HAL_UART_Transmit(&huart1, " - ", 3, 1000);	//Serial output to show user, current state

HAL_GPIO_WritePin(led_morse_GPIO_Port, led_morse_Pin, GPIO_PIN_SET);

HAL_Delay(3000);

HAL_GPIO_WritePin(led_morse_GPIO_Port, led_morse_Pin, GPIO_PIN_RESET);

HAL_Delay(1000);

return;

}

void space() {

HAL_Delay(1000);

HAL_UART_Transmit(&huart1, "   ", 3, 1000); //Serial output to show user, current state

return;

}

void ledMorseAlphabet(uint8_t input) {

switch (input) {

case 'A':

dot();space();dash();

break;

case 'B':

dash();space();dot();space();dot();space();dot();

break;

case 'C':

dash();space();dot();space();dash();space();dot();

break;

case 'D':

dash();space();dot();space();dot();space();

break;

case 'E':

dot();

break;

case 'F':

dot();space();dot();space();dash();space();dot();

break;

case 'G':

dash();space();dash();space();dot();

break;

case 'H':

dot();space();dot();space();dot();space();dot();

break;

case 'I':

dot();space();dot();

break;

case 'J':

dot();space();dash();space();dash();space();dash();

break;

case 'K':

dash();space();dot();space();dash();

break;

case 'L':

dot();space();dash();space();dot();space();dot();

break;

case 'M':

dash();space();dash();

case 'N':

dash();space();dot();

case 'O':

dash();space();dash();space();dash();space();

break;

case 'P':

dot();space();dash();space();dash();space();dot();

break;

case 'Q':

dash();space();dash();space();dot();space();dash();

break;

case 'R':

dot();space();dash();space();dot();

break;

case 'S':

dot();space();dot();space();dot();

break;

case 'T':

dash();

break;

case 'U':

dot();space();dot();space();dash();

break;

case 'V':

dot();space();dot();space();dot();space();dash();

break;

case 'W':

dot();space();dash();space();dash();

break;

case 'X':

dash();space();dot();space();dot();space();dash();

break;

case 'Y':

dash();space();dot();space();dash();space();dash();

break;

case 'Z':

dash();space();dash();space();dot();space();dot();

break;

Configurations:

Pin configuration:

USART DMA mode configuration:

Output:

Morse code output on LED: for dash led will be on for 3 seconds and for dot led will be on for 1 second